M DHANESH

ECA 1408   - EMBEDDED SYSTEM                                                           192512348

EXP:26

PROGRAM CODE:

#include <systemc>

#include <tlm>

#include <tlm_utils/simple_initiator_socket.h>

#include <tlm_utils/simple_target_socket.h>

#include <iostream>

using namespace sc_core;

using namespace tlm;

using namespace std;

// Target Module

struct Target : sc_module {

tlm_utils::simple_target_socket<Target> socket;

SC_CTOR(Target) : socket("socket") {

socket.register_b_transport(this, &Target::b_transport);

}

void b_transport(tlm_generic_payload& trans, sc_time& delay) {

int* data = reinterpret_cast<int*>(trans.get_data_ptr());

cout << "Target received: " << *data << endl;

delay += sc_time(10, SC_NS);

}

};

// Initiator Module

struct Initiator : sc_module {

tlm_utils::simple_initiator_socket<Initiator> socket;

SC_CTOR(Initiator) : socket("socket") {

SC_THREAD(thread_process);

}

void thread_process() {

tlm_generic_payload trans;

sc_time delay = SC_ZERO_TIME;

int data = 555;

trans.set_command(TLM_WRITE_COMMAND);

trans.set_address(0);

trans.set_data_ptr(

reinterpret_cast<unsigned char*>(&data));

trans.set_data_length(sizeof(data));

trans.set_streaming_width(sizeof(data));

trans.set_byte_enable_ptr(nullptr);

trans.set_dmi_allowed(false);

trans.set_response_status(TLM_INCOMPLETE_RESPONSE);

cout << "Initiator sending: " << data << endl;

socket->b_transport(trans, delay);

wait(delay);

}

};

int sc_main(int argc, char* argv[]) {

Initiator initiator("initiator");

Target target("target");

initiator.socket.bind(target.socket);

sc_start();

return 0;

}

OUTPUT: